Detailed Project Proposal

Arrhythmia Recognition Using Neural Networks: A Comparative Study

Student Name: Hariprasad Murugesan

Student ID: 24135993

Course: MSc Data Science and Analytics

Module code with name: 7COM1039-0509-2025 - Advanced Computer Science Masters Project

Supervisor: Neeli Khalid

1.Introduction

Cardiovascular disease remains the leading cause of death worldwide, accounting for an estimated 17.9 million deaths annually according to the World Health Organisation. A large proportion of these deaths are attributable to cardiac arrhythmias. Cardiac arrhythmias are electrical abnormalities within the heart that can cause severe clinical complications, including sudden cardiac death. The electrocardiogram (ECG) remains the primary non-invasive diagnostic tool for recording the heart’s electrical activity. While automated arrhythmia recognition is a well-established domain within machine learning, persistent challenges remain regarding clinical application. Specifically, real-world ECG signals are highly susceptible to noise and recording artifacts, heartbeat datasets exhibit severe class imbalance, and morphologically similar arrhythmia types frequently result in ambiguous class boundaries.

This project proposes a systematic comparative study of neural network architectures for ECG classification. By evaluating a Multi-Layer Perceptron (MLP), a 1D Convolutional Neural Network (1D-CNN), and a Long Short-Term Memory (LSTM) network, this research will investigate the transition from traditional feature-engineered connectionist models to advanced sequence-learning deep architectures. The primary objective is to determine which structural paradigm most effectively mitigates the inherent challenges of real-world arrhythmia data.

2. Research Questions

- What are the existing arrhythmia recognition techniques and what are their key limitations in handling real-world ECG data?

- How do Multi-Layer Perceptron (MLP), 1D-CNN, and LSTM architectures compare in classifying cardiac arrhythmias from MIT-BIH ECG signals, and to what extent does oversampling mitigate class imbalance effects on per-class sensitivity and overall classification performance?

- How does the application of class imbalance handling techniques, such as SMOTE, affect the performance of MLP, 1D-CNN, and LSTM architectures across minority arrhythmia classes in the MIT-BIH dataset?

- To what extent does the choice of feature representation like time-domain, frequency-domain, and wavelet-domain influence the classification performance of neural networks on ECG data?

3. Literature Review

The standard ECG waveform tracks cardiac activity, where deviations indicate dangerous arrhythmias that require robust automated classification. While classical machine learning models provide a solid baseline, they struggle with minority class imbalances and rely on manual feature extraction. Conversely, deep learning models (CNNs and RNNs) achieve state-of-the-art accuracy but act as computationally heavy, data-hungry "black boxes." A persistent challenge in this field is that benchmark datasets like MIT-BIH are heavily biased toward normal beats (~75%), and high inter-patient variability limits model generalization. To address these limitations, this project contrasts a standard feedforward Multi-Layer Perceptron (MLP) utilizing multi-domain features against advanced deep learning architectures specifically 1D-CNNs for spatial morphology and LSTMs for sequential temporal dependencies.

The following sources were analysed as they are most relevant to this project. Jahan, I. et al. (2025) built a CNN-LSTM-Attention model for ECG classification, showing that combining spatial and temporal feature extraction with an attention mechanism improves arrhythmia detection. This directly shapes the architectural choices made in this project. Sun, A. et al. (2024) took a similar approach, adding a channel attention module to a CNN-LSTM model and testing it on the MIT-BIH dataset, offering a useful benchmark to compare against. Thota, V. et al. (2025) went further by designing a lightweight version of this type of model that works on both standard 12-lead ECGs and single-lead wearable devices, proving that compact deep learning models can still perform well in real-world settings.

On the data side, Mirdan (2025) used SMOTE augmentation and Gaussian noise to handle class imbalance and was careful to avoid data leakage between patients during evaluation, which sets a useful methodological example for this project. Ahamed, R. et al. (2026) kept things simpler, applying an MLP with SMOTE on the same MIT-BIH dataset and still reaching 96.4% accuracy, making it a solid baseline to compare against. A 2025 systematic review in Frontiers in Digital Health confirmed that class imbalance remains the biggest unsolved problem in ECG classification, which supports the focus of this project.

While recent studies have made progress on individual architectures and some address class imbalance, most test only one model at a time and do not compare fairly under the same conditions. Few are careful about patient data leaking between training and test sets, which inflates reported accuracy. This project addresses that gap by comparing MLP, CNN-LSTM, and CNN-LSTM-Attention models under the same data, preprocessing, and evaluation conditions on the MIT-BIH dataset, with proper leakage controls, to find out which architectural features actually make a difference.

4. Project Aim

The aim of this project is to evaluate and compare three distinct neural network architectures (MLP, 1D-CNN, and LSTM) for the automated detection of cardiac arrhythmias using publicly available benchmark ECG data. The study will implement a reproducible pre-processing and evaluation pipeline to systematically benchmark these models. Ultimately, the project seeks to identify the architecture that best navigates the complexities of clinical ECG data like noise, extreme class imbalance, and overlapping decision boundaries while maintaining high diagnostic reliability.

5. Objectives

- Conduct a comprehensive literature review to identify the limitations of existing traditional machine learning and deep learning techniques in arrhythmia recognition.

- Acquire and pre-process benchmark ECG data from the MIT-BIH Arrhythmia Database, applying robust signal filtering and segmentation methodologies.

- Engineer multi-domain input representations, extracting time-domain, frequency-domain, and wavelet-domain features to serve as baselines against raw sequential inputs.

- Implement and optimise MLP, 1D-CNN, and LSTM architectures for multi-class classification.

- Evaluate network performance utilizing a rigorous framework encompassing overall accuracy, sensitivity, specificity, F1-score, and AUC-ROC.

- Analyse the efficacy of data balancing techniques (e.g., SMOTE) on minority class recognition rates across all three models.

- Perform detailed error analysis to map misclassification trends and evaluate failure modes.

- Synthesise findings to formulate evidence-based conclusions regarding optimal neural network designs for scalable clinical deployment.

6. Methodology

- Dataset: The primary source is the MIT-BIH Arrhythmia Database (48 half-hour ECG recordings from 47 subjects via PhysioNet), where Python-facilitated signal loading and parsing will map beats to five AAMI EC57 classes: Normal (N), Supraventricular (S), Ventricular (V), Fusion (F), and Unknown (Q).

- Pre-Processing: Signals will be denoised using a band-pass Butterworth filter (0.5–40 Hz), followed by R-peak identification via the Pan-Tompkins algorithm, temporal segmentation into fixed windows (300 ms prior to and 400 ms following each peak), and per-beat Z-score standardisation for stable gradient descent.

- Feature Engineering & Architecture: A standard Multi-Layer Perceptron (MLP) baseline will classify engineered features (time-domain metrics, FFT variables, and DWT coefficients) reduced via PCA to retain 95% variance, whereas 1D-Convolutional Neural Networks (1D-CNN) and Long Short-Term Memory (LSTM) networks will ingest raw, fixed-length voltage sequences to autonomously extract spatial, morphological, and long-range temporal dependencies.

- Optimisation & Imbalance Handling: All architectures will undergo cross-validated grid search for hyperparameter tuning, while the dataset's profound imbalance (~75% normal beats) will be mitigated using SMOTE for the engineered baseline and cost-sensitive class-weighted loss functions for the deep learning models.

- Evaluation Framework: Model performance will be assessed using a stratified 10-fold cross-validation strategy, tracking per-class sensitivity/specificity, macro-averaged F1-Score, AUC-ROC, Cohen’s Kappa, and confusion matrices, with statistical significance determined by a paired Wilcoxon signed-rank test ($\alpha = 0.05$).

7. Expected Outcomes & Deliverables

- A robust, reproducible Python codebase encompassing the full pipeline (from raw signal processing to model evaluation), released on GitHub under an open-source licence.

- A comprehensive empirical comparison between traditional feature-engineering approaches (MLP) and automated representation learning (CNN, LSTM).

- Evidence-based insights detailing which architecture most effectively resolves the inherent noise, imbalance, and class overlap typical of clinical ECG data.

- An analysis of the efficacy of distinct feature representation domains (time, frequency, wavelet) versus raw sequential inputs.

- A complete master's dissertation inclusive of full methodology, quantitative results, error analysis, and reproducible code documentation.

8. Project Plan

| Phase / Task | W1 | W2 | W3 | W4 | W5 | W6 | W7 | W8 | W9 | W10 | W11 | W12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase 1 Literature Review & Proposal Finalisation |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 2 Dataset Acquisition & Pre-processing |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 3 Feature Engineering & Formatting |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 4 Architecture Implementation & Tuning |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 5 Comparative Evaluation & Imbalance Analysis |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 6 Error Analysis & Interpretation |  |  |  |  |  |  |  |  |  |  |  |  |
| Phase 7 Dissertation Write-up & Code Release |  |  |  |  |  |  |  |  |  |  |  |  |

9. Resources Required

- Dataset: MIT-BIH Arrhythmia Database (freely accessible via PhysioNet: physionet.org/content/mitdb/).

- Software: Python 3.10+ utilizing libraries such as NumPy, SciPy, scikit-learn, PyWavelets, WFDB, imbalanced-learn (for SMOTE), and deep learning frameworks (e.g., PyTorch or TensorFlow), alongside Matplotlib and Seaborn for visualisation.

- Literature: Academic database access (IEEE Xplore, PubMed, ScienceDirect) provided by the University library.

10. References

- Ahamed, R. et al. (2026) 'An augmented ECG data-based classification for arrhythmia using optimal feature set', Health Information Science and Systems, 14(1). doi:10.1007/s13755-026-00440-3.

- Frontiers in Digital Health (2025) 'Advances in machine and deep learning for ECG beat classification: a systematic review', Frontiers in Digital Health, 7, art. 1649923. doi:10.3389/fdgth.2025.1649923.

- Hannun, A.Y. et al. (2019) 'Cardiologist-level arrhythmia detection and classification using a deep neural network', Nature Medicine, 25(1), pp. 65–69.

- Jahan, I. et al. (2025) 'Enhancing cardiac arrhythmia detection: deep learning-based CNN-LSTM-attention framework for digitized ECG classification', in Bhattacharya, A. et al. (eds) Innovations in Data Analytics – ICIDA 2024. Lecture Notes in Networks and Systems, vol. 1408. Singapore: Springer. doi:10.1007/978-981-96-6297-5_19.

- Kiranyaz, S., Ince, T. and Gabbouj, M. (2016) 'Real-time patient-specific ECG classification by 1-D convolutional neural networks', IEEE Transactions on Biomedical Engineering, 63(3), pp. 664–675.

- Mirdan, A.S. (2025) 'A deep attention-based model on MIT-BIH', Journal of Al-Qadisiyah for Computer Science and Mathematics, 17(2), pp. 29–47.

- Moody, G.B. and Mark, R.G. (2001) 'The impact of the MIT-BIH Arrhythmia Database', IEEE Engineering in Medicine and Biology Magazine, 20(3), pp. 45–50.

- PhysioNet (2024) MIT-BIH Arrhythmia Database. Available at: https://physionet.org/content/mitdb/1.0.0/ (Accessed: 1 June 2026).

- Sun, A. et al. (2024) 'An arrhythmia classification model based on a CNN-LSTM-SE algorithm', Sensors, 24(19), art. 6306. doi:10.3390/s24196306.

- Thota, V. et al. (2025) 'A lightweight CNN-attention-BiLSTM architecture for multi-class arrhythmia classification on standard and wearable ECGs', in Proceedings of CISP-BMEI 2025. arXiv:2511.08650.

- World Heart Federation (2023) World Heart Report 2023: Confronting the Heart Failure Pandemic. Geneva: World Heart Federation.